Lab 9 – Series/Parallel Resistors

Names: Ivan Cervantes

Date: 10/22/2015

The purpose of this lab is to:

Experiment with series circuits and verify that the simulation, analysis (calculations) and test results all agree.

From the resistor kit select 8 resistors: 2 each 470Ω, 2 each 1kΩ, and 1 each of the following:

2.2kΩ, 3.3kΩ, 4.7kΩ, 10kΩ

Measure and record the value of each resistor.  Connect the resistors as shown in Figure 1. Measure and record the total resistance, RT.  Then connect the resistors as shown in Figure 2, the 9V come from the Elvis II (Modular Engineering Educational Laboratory Platform).  Then measure and record with the Digital Multimeter the current and voltages of the series circuit.

Figure 1

Series/Parallel Circuit

Equipment needed:

1 – Digital Multimeter

1 – Elvis II

8 – resistors.

|  | Expected | Measured |
| --- | --- | --- |
| R1 = | 470Ω | 461.2Ω |
| R2 = | 470Ω | 462.5Ω |
| R3 = | 1.0E+3Ω | 977.8Ω |
| R4 = | 1.0E+3Ω | 983.3Ω |
| R5 = | 2.2E+3Ω | 2.18E+3Ω |
| R6 = | 3.3E+3Ω | 3.218E+3Ω |
| R7 = | 4.7E+3Ω | 4.61E+3Ω |
| R8 = | 10.0E+3Ω | 9.776E+3Ω |

Expected = value you expect it to be

Measured = using Digital Multimeter

|  | Calculated | Simulated | Measured |
| --- | --- | --- | --- |
| R34 = | 500Ω | 500Ω | 470Ω |
| R567 = | 1030.565Ω | 1.031kΩ | 928Ω |
| R345678 = | 11530.565Ω | 11.531kΩ | 11.285E+3Ω |
| R2345678 = | 451.593Ω | 451.593Ω | 444Ω |
| RT = | 921.593Ω | 921.593Ω | 906Ω |

Calculated = using Excel calculations

Simulated = Multisim simulation

Measured = using Digital Multimeter

Figure 2

Series/Parallel Circuit

|  | Calculated | Simulated | Measured |
| --- | --- | --- | --- |
| V1 = | 9V | 9V | 9.161 |
| IT = | 98.8E-3A | 9.77mA | 9.997mA |
| VA = | 4.41V | 4.41V | 4.4955 |
| VB = | 4.12V | 4.22V | 4.2998 |
| VC = | 1.064V | 3.82V | 3.8946 |

Calculated = using Excel calculations

Simulated = Multisim simulation

Measured = using Digital Multimeter

Adjust R2 so that the VA voltage is equal to 4.5V. Then measure the value of the new R2 and calculate and simulate a value that would produce the 4.5V.

|  | Measured | Simulated | Calculated |
| --- | --- | --- | --- |
| VA = | 4.5 | 4.5 | 4.5021 |
| R2 = | 462 | 490 | 470 |

Calculated = using Excel calculations

Simulated = Multisim simulation

Measured = using Digital Multimeter

Observations: This lab was just a compilation of everything we had learned in the past eight weeks. It ranged from calculating voltages, currents, and resistances to going into the lab, building circuits, and measuring them. I found that I was a bit rusty in my calculations for voltages across the circuit, other than that I was pretty confident in my abilities.