Lab 10 – Series/Parallel Capacitors

Names: Ivan Cervantes, Zackary Reigelman, Pavlo

Date: 11/5/15

The purpose of this lab is to:

Experiment with series circuits and parallel combinations of capacitors.

The following capacitors are needed (1 each of the following): 10uF, 22uF and 47uF

Measure and record the capacitance of each capacitor using the LCR meter.  Connect the capacitors as shown in Figure 1 and measure and record the total capacitance, CT.  Then connect the capacitors as shown in Figure 2 and measure and record the total capacitance, CT.

Equipment needed:

1 – LCR Meter

Figure 1

Series Circuit

1 – Elvis II

3 – capacitors

|  | Expected | Measured |
| --- | --- | --- |
| C1 = | 10uF | 9.761uF |
| C2 = | 22uF | 21.773uF |
| C3 = | 47uF | 46.935uF |
| CT = | 5.99768uF | 5.893uF |

Expected = value you expect it to be

Measured = using LCR Meter

|  | Expected | Measured |
| --- | --- | --- |
| C1 = | 10uF | 9.761uF |
| C2 = | 22uF | 21.773uF |
| C3 = | 47uF | 46.935uF |
| CT = | 79uF | 78.47uF |

Expected = value you expect it to be

Measured = using LCR Meter

Figure 2

Parallel Circuit

Observations: Calculating the capacitance of capacitors in a series format uses the same formula as calculating the resistance of resistors in a parallel format. It is the same scenario for calculating the capacitance of capacitors in a parallel format.