Lab8 – Black Box 3 Design

Names: Ivan Cervantes, Zackary Reigelman, Pavlo

Date: 10/15/15

The purpose of this lab is to:

Learn about building a circuit that produces exactly 1.3V

Using at least 3 equal value resistors (in the Black Box) design a circuit that produces an output voltage of 1.3V. Then adjust R1 so that the output voltage is exactly 1.3V.

Equipment needed:

1 – Digital Multimeter

1 – Elvis II

5 – Standard Resistors

1 – 5 Kohm pot

|  | Measured | Calculated | Simulated |
| --- | --- | --- | --- |
| V1 = | 8.975 V | 9 V | 9V |
| VA = | 5.148 V | 5.14 V | 5.14 V |
| VB = | 3.8267 V | 3.86 V | 3.86 V |
| VA - VB = | 1.3213 V | 1.28 V | 1.28 V |
| (VA - VB) adj = | 1.303 V | 1.3 V | 1.3 V |

|  | Design | Measured |
| --- | --- | --- |
| R1 = | 4.7 kohm | 4.61 kohm |
| R2 = | 4.7 kohm | 4.62 kohm |
| R3 = | 4.7 kohm | 4.73 kohm |
| R4 = | 4.7 kohm | 4.63 kohm |
| R5 = | 4.7 kohm | 4.58 kohm |
| R(Black Box) = | 1.567 kohm | 1.55 kohm |
| R1adj = | 4.58 kohm | 4.585 kohm |

Observations: The resistors in the black box ended up being 3 4.7 kohm resistors in parallel form, making the circuit a series-parallel circuit. We found that the R1 had to be adjusted to 4.58 kohm in order to achieve 1.3 volts on the multimeter.