Lab 12 – Series/Parallel Inductors

Names: Ivan Cervantes, Zackary Riegelman, Pavlo

Date: 12/3/2015

The purpose of this lab is to:

Experiment with series circuits and parallel combinations of inductors.

The following inductors are needed (1 each of the following): 1mH, 2.2mH and 4.7mH

Measure and record the inductance of each inductor using the LCR meter.  Connect the inductors as shown in Figure 1 and measure and record the total inductance, LT.  Then connect the inductors as shown in Figure 2 and measure and record the total inductance, LT.

Equipment needed:

1 – LCR Meter

Figure 1

Series Circuit

1 – Elvis II

3 – Inductors

Error! Not a valid link.

Expected = value you expect it to be

Simulated = using Multisim

Measured = using LCR Meter

|  | Expected | Simulated | Measured |
| --- | --- | --- | --- |
| L1 = | 1 mH | 1 mH | 1.0047 mH |
| L2 = | 2.2 mH | 2.2 mH | 2.1816 mH |
| L3 = | 4.7 mH | 4.7 mH | 4.3493 mH |
| LT = | 7.9 mH | 7.9 mH | 7.8757 mH |

Expected = value you expect it to be

Simulated = using Multisim

Measured = using LCR Meter

Figure 2

Parallel Circuit

|  | Expected | Simulated | Measured |
| --- | --- | --- | --- |
| L1 = | 1 mH | 1 mH | 1.0047 mH |
| L2 = | 2.2 mH | 2.2 mH | 2.1816 mH |
| L3 = | 4.7 mH | 4.7 mH | 4.3493 mH |
| LT = | .599 mH | .599 mH | .60870mH |

Expected = value you expect it to be

Simulated = using Multisim

Measured = using LCR Meter

Observations: Inductors require some resistance in order to be measured in multisim. Inductors act the exact same way as resistors do when they are in series and parallel circuits. This means that the excel calculation are also the same