Lab 3 – Series Resistors

Names: Brandon Steup, Corey Townsend

Date: _____________________

The purpose of this lab is to:

Experiment with series circuits and verify that the simulation, analysis (calculations) and test results all agree.

From the resistor kit select 3 resistors (10K, 2.2K, 4.7K)

Measure and record the value of each resistor.  Connect the resistors as shown in Figure 1. Measure and record the total resistance, RT. Then connect the resistors as shown in Figure 2, the 9V come from the Elvis II (Modular Engineering Educational Laboratory Platform). Then measure and record with the Digital Multimeter the current and voltages of the series circuit.

Equipment needed:

1 – Digital Multimeter

1 – Elvis II

3 – resistors.

Figure 1

|  | Measured | Calculated | Simulated |
| --- | --- | --- | --- |
| R1 = | 10035 | 10000 | 10k |
| R2 = | 2184 | 2200 | 2.2k |
| R3 = | 4643 | 4700 | 4.7k |
| RT = | 16859 | 16900 | 16.9k |

Measured = using Digital Multimeter

Calculated = based on color code and Excel values

Simulated = Multisim simulation

|  | Measured | Calculated | Simulated |
| --- | --- | --- | --- |
| IT = | 501.88*10^-6 | 532*10^-6 | 532*10^-6 |
| V1 = | 8.97 | 9 | 9 |
| VA = | 3.64 | 3.67 | 3.67 |
| VB = | 2.49 | 2.5 | 2.5 |

Measured = using Digital Multimeter

Calculated = based on color code and Excel values

Simulated = Multisim simulation

Figure 2

Observations: The resistors had some inconstancy that was expected when we measured them to verify that they were not defective and within spec. I also noticed that the over the course of measuring from It to VA there was a large drop in measured voltage consistent with the simulated results, along with another drop-in voltage when measured at the second point.  These observations are constant with Kirchoff’s voltage law.